Connect raspberry pi to aws iot using mqtt

Steps

- Login to Amazon AWS cloud

- Services ->  select  iot core

3. →  onboard

4. Get started

5. Select

Choose a platform - Linux

Choose a AWS IoT Device SDK -  Python

→ Next

Name → my_thing1

You can give any name to your thing

→ next Step

6.	Download the sdk

7. Download the certificates

8. Click on activate

9. Download the AWSiotpythonsdk using the command

$ pip install AWSIoTPythonSDK

10. There are 2 steps to download the root certificate

i)

Unzip the downloaded file

U will have the following files after unzipping

Give execute permissions to the start.sh file

By typing command in the command line

$ sudo +chmod ./start.sh

Execute the start.sh file

$ ./start.sh

ii) if the root certificate is not downloaded then  go to the link

https://docs.aws.amazon.com/iot/latest/developerguide/managing-device-certs.html#server-authentication

Select  Amazon Root CA 1

Copy the contents and create a new file in the folder where all the other keys are dowloaded

Save the file as root-CA.crt     extension

Note: all the keys and the root certificate  should be one folder.

To create an AWS IoT policy:

In the left navigation pane, choose Secure, and then Policies. On the You don't have a policy yet page, choose Create a policy.

secure→ certificate → right click on certificate → attach a policy

Attach the policy to certificate

Attach the certificate to the registered thing

Save the following two python scripts in the same folder where keys are downloaded

Aws_iot_pub.py

#!/usr/bin/python

############################################ 
# PROBLEM STATEMENT:
# This program will publish test mqtt messages using the AWS IOT hub
# 
# To test this program you have to run first its companinon aws_iot_sub.py
# that will subscribe and show all the messages sent by this program
#
############################################

############################################
# STEPS:
#
# 1. Sign in to AWS Amazon > Services > AWS IoT > Settings > copy Endpoint
#    This is your awshost
# 
# 2. Change following things in the below program:
#    a. awshost   (from step 1)
#    b. clientId  (Thing_Name)
#    c. thingName (Thing_Name)
#    d. caPath    (root-CA_certificate_Name)
#    e. certPath  (<Thing_Name>.cert.pem)
#    f. keyPath   (<Thing_Name>.private.key)
# 
# 3. Paste aws_iot_pub.py & aws_iot_sub.py python scripts in folder where all unzipped aws files are kept. 
# 4. Provide Executable permition for both the python scripts.
# 5. Run aws_iot_sub.py script
# 6. Run this aws_iot_pub.py python script
#
############################################

# importing libraries
import paho.mqtt.client as paho
import os
import socket
import ssl
from time import sleep
from random import uniform
 
connflag = False
 
def on_connect(client, userdata, flags, rc):                # func for making connection
    global connflag
    print "Connected to AWS"
    connflag = True
    print("Connection returned result: " + str(rc) )
 
def on_message(client, userdata, msg):                      # Func for Sending msg
    print(msg.topic+" "+str(msg.payload))
 
#def on_log(client, userdata, level, buf):
#    print(msg.topic+" "+str(msg.payload))
 
mqttc = paho.Client()                                       # mqttc object
mqttc.on_connect = on_connect                               # assign on_connect func
mqttc.on_message = on_message                               # assign on_message func
#mqttc.on_log = on_log

#### Change following parameters #### 
awshost = "xxxxxxxxxxxxxxxxxxx.amazonaws.com"      # Endpoint
awsport = 8883                                              # Port no.   
clientId = "thing_name"                                     # Thing_Name
thingName = "thing_name"                                    # Thing_Name
caPath = "/home/pi/Downloads/AmazonRootCA1.pem"                                      # Root_CA_Certificate_Name
certPath = "/home/pi/Downloads/3016e27518-certificate.pem.crt"                            # <Thing_Name>.cert.pem
keyPath = "/home/pi/Downloads/3016e27518-private.pem.key"                          # <Thing_Name>.private.key
 
mqttc.tls_set(caPath, certfile=certPath, keyfile=keyPath, cert_reqs=ssl.CERT_REQUIRED, tls_version=ssl.PROTOCOL_TLSv1_2, ciphers=None)  # pass parameters
 
mqttc.connect(awshost, awsport, keepalive=60)               # connect to aws server
 
mqttc.loop_start()                                          # Start the loop
 
while 1==1:
    sleep(5)
    if connflag == True:
        tempreading = uniform(20.0,25.0)                        # Generating Temperature Readings 
        mqttc.publish("temperature", tempreading, qos=1)        # topic: temperature # Publishing Temperature values
        print("msg sent: temperature " + "%.2f" % tempreading ) # Print sent temperature msg on console
    else:
        print("waiting for connection...")

Aws_iot_sub.py

#!/usr/bin/python

############################################ 
# PROBLEM STATEMENT:
#
# This program will subscribe and show all the messages sent by its companion 
# aws_iot_pub.py using AWS IoT hub
#
############################################

############################################
# STEPS:
#
# 1. Sign in to AWS Amazon > Services > AWS IoT > Settings > copy Endpoint
#    This is your awshost
# 
# 2. Change following things in the below program:
#    a. awshost   (from step 1)
#    b. clientId  (Thing_Name)
#    c. thingName (Thing_Name)
#    d. caPath    (root-CA_certificate_Name)
#    e. certPath  (<Thing_Name>.cert.pem)
#    f. keyPath   (<Thing_Name>.private.key)
# 
# 3. Paste aws_iot_pub.py & aws_iot_sub.py python scripts in folder where all unzipped aws files are kept. 
# 4. Provide Executable permition for both the python scripts.
# 5. Run aws_iot_sub.py script
# 6. Run this aws_iot_pub.py python script
#
############################################

# importing libraries
import paho.mqtt.client as paho
import os
import socket
import ssl
 
def on_connect(client, userdata, flags, rc):                # func for making connection
    print("Connection returned result: " + str(rc) )
    # Subscribing in on_connect() means that if we lose the connection and
    # reconnect then subscriptions will be renewed.
    client.subscribe("#" , 1 )                              # Subscribe to all topics
 
def on_message(client, userdata, msg):                      # Func for receiving msgs
    print("topic: "+msg.topic)
    print("payload: "+str(msg.payload))
 
#def on_log(client, userdata, level, msg):
#    print(msg.topic+" "+str(msg.payload))
 
mqttc = paho.Client()                                       # mqttc object
mqttc.on_connect = on_connect                               # assign on_connect func
mqttc.on_message = on_message                               # assign on_message func
#mqttc.on_log = on_log

#### Change following parameters ####  
awshost = "xxxxxxxxxxxxxxxxxxxxxxxxxxxx.amazonaws.com"      # Endpoint
awsport = 8883                                              # Port no.   
clientId = "thing_name"                                     # Thing_Name
thingName = "thing_name"                                    # Thing_Name
caPath = "/home/pi/Downloads/AmazonRootCA1.pem"                                      # Root_CA_Certificate_Name
certPath = "/home/pi/Downloads/3016e27518-certificate.pem.crt"                            # <Thing_Name>.cert.pem
keyPath = "/home/pi/Downloads/3016e27518-private.pem.key"                          # <Thing_Name>.private.key
                       # <Thing_Name>.private.key
 
mqttc.tls_set(caPath, certfile=certPath, keyfile=keyPath, cert_reqs=ssl.CERT_REQUIRED, tls_version=ssl.PROTOCOL_TLSv1_2, ciphers=None)      # pass parameters
 
mqttc.connect(awshost, awsport, keepalive=60)               # connect to aws server
 
mqttc.loop_forever()                                        # Start receiving in loop

Give execute permission to both the files

Change    excute  to  → Anyone

Do it for both the files

Run both files

To check on the aws iot

In the navigation pane → Test →

Subscribe to all the topics   i.e.  #

Click  subscribe